Arab Academy for Science, Technology

& Maritime Transport

College of Engineering — Computer Engineering Department

PROJECT DOCUMENTATION

Smart Adaptive Street Lighting System

with AI-Based Vehicle Detection

An embedded vision system that intelligently illuminates streets by detecting vehicles in real time and propagating responsive light waves across nine simulated street lamps.

Course: Embedded Systems

Supervisor: Dr. Amr Fahmy

Semester 8 — Academic Year 2025 / 2026

Submission Date: June 7, 2026

# 1. Team Information

This project was developed by a three-member team from the Computer Engineering Department at AAST. Each member owned an end-to-end vertical of the system, while integration and code reviews were carried out jointly.

| Name | Student ID | Role |
| --- | --- | --- |
| Ahmed Khaled Bayoumy | 221006382 | Team Lead — Controller & Logic |
| Ahmed Essam Ahmed | 221004518 | Hardware Integration & Maquette |
| Mohamed Moatassem | 221005418 | Computer Vision & AI Detection |

## Sub-group distribution

- Controller & Logic — STM32 Blue Pill firmware (Arduino C++), multi-wave engine, LDR day/night state machine, UART command parser, system integration. (Ahmed Khaled Bayoumy)

- Hardware Integration — Schematic design, breadboard wiring, power architecture, maquette construction, ST-Link flashing workflow. (Ahmed Essam Ahmed)

- Computer Vision & AI — ESP32-CAM firmware (WiFi camera server, flash PWM control), Python YOLOv8 pipeline with ByteTrack object tracking and adaptive scene-brightness logic. (Mohamed Moatassem)

# 2. Introduction & Problem Statement

## Background

Conventional street lighting either burns continuously through the night at full brightness, wasting energy, or relies on coarse motion sensors that trigger only the single lamp directly beneath an approaching vehicle. Neither approach offers a driver-aware, predictive lighting experience. Smart adaptive lighting is a recognised area of urban-infrastructure research, and the recent availability of low-cost camera modules and embedded AI inference has opened the door to vision-based traffic awareness on classroom budgets.

## Problem Statement

- Static street lights consume the same power whether the road is empty or busy, leading to enormous nightly energy waste in low-traffic hours.

- Simple PIR-based adaptive lights illuminate one lamp at a time and do not anticipate the driver's path, leaving the road ahead dark.

- Camera-based traffic systems on the market are expensive industrial units; no comparable affordable demonstration platform exists for academic use.

- There is no clean, reproducible example combining a microcontroller, an AI-capable camera module, and a host inference engine in a single educational project.

## Motivation

Our motivation is to prove that a fully working adaptive street-light system — including real-time AI detection — can be built using ~150 EGP worth of hobby hardware, an STM32 Blue Pill, an ESP32-CAM, and a standard laptop. This shows the principle behind smart-city lighting infrastructure while producing a hands-on educational artefact for the Embedded Systems course.

# 3. Objectives

The objectives below define the scope of the project and were used as acceptance criteria during testing.

- Design and implement a nine-LED street-lamp simulation driven by an STM32 Blue Pill microcontroller.

- Detect day and night ambient conditions automatically using an LDR voltage divider on the controller's ADC.

- Capture live video over WiFi from an ESP32-CAM module mounted at the entry of the road.

- Run real-time object detection on the host laptop using YOLOv8-nano with ByteTrack object tracking.

- Fire one — and only one — wave per uniquely tracked object, regardless of motion or partial occlusion.

- Support multiple overlapping waves so that a second vehicle entering during an existing wave produces a second independent wave.

- Automatically control the camera's flash LED based on measured scene brightness, with cooldown to prevent flicker.

- Deliver the system as a maquette using nine architectural-scale street lamps mounted on a foam-board road.

- Document every subsystem and test case rigorously to satisfy the Embedded Systems project rubric.

- Provide a clean migration path from Arduino Uno to STM32 Blue Pill, demonstrating cross-platform embedded portability.

# 4. Technology Stack

The technology stack was selected to balance teaching value, real-time performance, and component cost. Each line item below is justified later in the integration section.

| Layer | Technology | Purpose |
| --- | --- | --- |
| Microcontroller | STM32 Blue Pill (STM32F103C8T6) | Main wave controller and state machine |
| Vision Module | ESP32-CAM (AI Thinker, OV2640 sensor) | Camera streaming and flash control |
| AI Framework | Ultralytics YOLOv8-nano + ByteTrack | Real-time detection and ID-stable tracking |
| Programming Languages | Arduino C++ (both MCUs); Python 3 | Firmware and host pipeline |
| Host Libraries | OpenCV, NumPy, requests, PySerial | Frame I/O and image handling |
| IDE / Toolchain | Arduino IDE 2.x with STM32 core | Building and uploading firmware |
| Programmer | ST-Link V2 via SWD | Flashing STM32 (workaround for broken USB) |
| Sensors / Inputs | LDR (light-dependent resistor) + 10 kΩ divider | Day/night detection |
| Actuators / Outputs | 9 × 5 mm LEDs + 220 Ω resistors | Street-lamp wave animation |
| Communication | UART 9600 baud (ESP32 GPIO1 → STM32 PA10) | Detection-trigger channel |
| Power | 5 V wall adapter (camera rail) + ST-Link 3.3 V (controller) | Two-rail isolated power design |
| Maquette Components | Scale architectural street lamps, foam-board base | Physical demonstration model |

# 5. System Architecture

## High-Level Diagram

The system is composed of three independent tiers communicating in sequence:

Tier 1 — Vision (ESP32-CAM): captures JPEG frames at QVGA resolution and exposes them over HTTP through a built-in WiFi web server.

Tier 2 — Inference (Host Laptop): runs Python with Ultralytics YOLOv8-nano, performs object tracking with ByteTrack, and decides whether each frame represents a new object entering the scene.

Tier 3 — Actuation (STM32 Blue Pill): receives the textual 'CAR' command over UART from the camera, then propagates a multi-step wave across the nine LEDs while continuously polling the LDR for night-mode override.

## Data Flow

- Vehicle enters camera field of view → ESP32-CAM captures the frame.

- Laptop fetches the frame via HTTP GET on /jpg and feeds it to YOLOv8.

- YOLO produces bounding boxes with stable tracking IDs; previously fired IDs are remembered and ignored.

- On a new ID, the Python script issues an HTTP GET on /car to the ESP32.

- ESP32 receives the request and sends the literal text 'CAR\n' on its UART (GPIO1).

- STM32 reads the line on Serial1 (PA10), parses it, and — only if it is currently in NIGHT mode — spawns a new wave.

- Each wave is an independent state machine; up to ten can run concurrently.

- The wave lights LEDs 1 → 9 sequentially with a 350 ms step, holding each pulse for ~240 ms; lamps return to dim mode afterwards.

## Controller Internals

The Blue Pill firmware is built around two data structures: a Wave array (active flag, current LED index, last step timestamp) and a PulseLED array (active flag and off-time per LED). Each main-loop iteration: (1) reads the LDR, (2) checks Serial1 for incoming commands, (3) advances every active wave, (4) expires any pulses whose deadline has passed, and (5) renders the LED state. Software PWM is used to produce the dim ambient glow on non-PWM-capable pins.

## Camera Server Internals

The ESP32-CAM firmware registers four HTTP endpoints on port 80: /jpg returns the most recent camera frame; /car triggers a UART 'CAR' message to the Blue Pill; /flash/on and /flash/off control the GPIO4 flash LED via PWM at 5 kHz. The 5 V supply for the module is provided by a dedicated wall adapter, isolating the camera's WiFi current bursts from the controller rail.

# 6. Development Phases

The project was executed in nine sequential phases. Each phase was bounded by a working deliverable that could be demonstrated before moving on.

| Phase | Title | Deliverable |
| --- | --- | --- |
| Phase 1 | Requirements & Planning | Drafted system requirements, selected components, defined acceptance criteria, sketched the system block diagram. |
| Phase 2 | Controller Prototype on Arduino Uno | Built initial nine-LED wave logic, LDR day/night switching, and serial-trigger interface on Arduino Uno to validate the algorithm. |
| Phase 3 | Camera Subsystem & Motion Detection | Programmed the ESP32-CAM with frame-difference motion detection over UART. Validated that real motion produces 'CAR' signals. |
| Phase 4 | Migration to AI Pipeline | Replaced motion detection with YOLOv8 object detection running on the host. Added WiFi streaming and HTTP control endpoints. |
| Phase 5 | Port to STM32 Blue Pill | Remapped GPIO definitions from Arduino Uno to STM32, switched UART from Serial to Serial1, adjusted ADC scaling for the 12-bit STM32 ADC. |
| Phase 6 | Hardware Power Redesign | Diagnosed brownout symptoms; introduced a dedicated 5 V wall adapter for the camera, established a shared ground bus across all subsystems. |
| Phase 7 | Maquette Assembly & Integration | Mounted nine architectural-scale street lamps on a foam-board road; routed wiring underneath; integrated the LDR and camera mount. |
| Phase 8 | Testing & Verification | Executed the 10-case test plan; recorded results; tuned LDR threshold and detection persistence frames for the demo environment. |
| Phase 9 | Documentation & Presentation | Produced this documentation, the evaluation rubric annex, and the defence presentation deck. |

# 7. Task Distribution

Each member owned one technical vertical end-to-end. Joint integration tasks were performed as a team to ensure all three subsystems communicated correctly.

| Member | Role | Tasks Performed | Status |
| --- | --- | --- | --- |
| Ahmed Khaled Bayoumy | Controller & Logic | Designed the multi-wave state machine; wrote the STM32 Blue Pill firmware; tuned LDR thresholds and timing constants; defined the serial protocol; coordinated subsystem interfaces. | Completed |
| Ahmed Essam Ahmed | Hardware Integration | Designed the schematic; assembled the breadboard; resolved the brownout by adding a dedicated 5 V adapter; built the maquette with the nine scale lamps; managed ST-Link flashing. | Completed |
| Mohamed Moatassem | Computer Vision & AI | Wrote the ESP32-CAM firmware with WiFi server and flash control; built the Python YOLOv8 pipeline with ByteTrack; implemented adaptive flash logic based on scene brightness. | Completed |
| Joint | Integration & Test | Wired the maquette; verified the full pipeline end-to-end; produced the test results table; recorded the demo video; built the presentation deck. | Completed |

# 8. Project Timeline

The project was executed across a nine-week academic window during Semester 8 of the 2025/2026 academic year.

| Week | Milestone |
| --- | --- |
| Week 1 | Component sourcing, account setup, system block diagram, GitHub repository creation. |
| Week 2 | Arduino Uno prototype: nine-LED wave engine, LDR day/night logic, serial trigger from Serial Monitor. |
| Week 3 | ESP32-CAM motion detection firmware, frame-difference algorithm, UART output to Arduino. |
| Week 4 | WiFi server on ESP32-CAM; Python YOLOv8-nano environment setup on Mac (virtualenv, ultralytics, OpenCV). |
| Week 5 | Full AI pipeline: ByteTrack persistence, per-ID firing, scene-brightness flash control. |
| Week 6 | Port of firmware to STM32 Blue Pill; ADC re-scaling; serial port reassignment to Serial1; ST-Link upload workflow. |
| Week 7 | Power redesign with dedicated 5 V adapter; brownout debugging; common-ground bus. |
| Week 8 | Maquette construction with scale street lamps; wiring routing; LDR mount; final integration. |
| Week 9 | Test plan execution, documentation, rubric preparation, presentation deck, dry-run demo. |

# 9. Full Project Documentation

## 9.1 Final Product Features

- Real-time AI-based object detection: identifies vehicles or any of 80 COCO object classes passing through the camera's field of view.

- Stable per-object tracking: each unique entity triggers exactly one wave, even under partial occlusion or motion.

- Multi-wave concurrency: up to 10 overlapping waves can propagate simultaneously across the nine lamps.

- Automatic day/night switching: LDR voltage divider on STM32 PA0 selects between OFF (day), DIM (night idle), and FULL-PULSE (wave) states.

- Adaptive camera flash: scene brightness is measured every 30 frames; flash auto-toggles with a 3-second cooldown to avoid flicker.

- Remote camera control: HTTP endpoints expose /flash/on, /flash/off, /car (manual trigger), and /jpg (latest frame).

- WiFi-decoupled architecture: the camera and controller share no copper signal — only the UART trigger line — eliminating ground loops.

- Cross-platform firmware: the same wave engine compiles on Arduino Uno and STM32 Blue Pill with only pin macro changes.

- Software PWM dim mode: provides smooth ambient glow on non-PWM pins, enabling the use of all nine LED channels.

- Maquette presentation: nine architectural-scale street lamps mounted on a black foam-board road with white lane markings.

## 9.2 Non-Functional Requirements

- Performance — End-to-end detection latency ≤ 300 ms (camera → YOLO → ESP32 → STM32).

- Reliability — WiFi auto-reconnect on the ESP32; controller continues night-mode dim even if the camera drops offline.

- Power Safety — Dedicated 5 V supply for the camera; controller draws less than 100 mA on the 3.3 V rail.

- Maintainability — Three independent codebases with clear boundaries; tuning constants centralised at the top of each file.

- Portability — Firmware ports between Arduino Uno and STM32 Blue Pill with only pin macro edits.

- Demonstrability — Operates correctly under typical classroom lighting; LDR threshold calibrated for the actual demo room.

## 9.3 HTTP Endpoints Exposed by the ESP32-CAM

| Method | Path | Behaviour |
| --- | --- | --- |
| GET | /jpg | Returns the most recent JPEG frame from the camera (QVGA, JPEG quality 12). |
| GET | /car | Triggers the UART 'CAR\n' message to the STM32 controller, spawning a wave when in NIGHT mode. |
| GET | /flash/on | Turns the GPIO4 flash LED ON at ~30% PWM duty (value 80 / 255). |
| GET | /flash/off | Turns the flash LED OFF (PWM duty 0). |

## 9.4 STM32 Pin Assignments

| Pin | Mode | Function |
| --- | --- | --- |
| PB0 | OUTPUT | LED 1 — first street lamp |
| PB1 | OUTPUT | LED 2 |
| PB3 | OUTPUT | LED 3 |
| PB4 | OUTPUT | LED 4 |
| PB5 | OUTPUT | LED 5 |
| PB6 | OUTPUT | LED 6 |
| PB7 | OUTPUT | LED 7 |
| PB8 | OUTPUT | LED 8 |
| PB9 | OUTPUT | LED 9 — last street lamp |
| PA0 | ANALOG_IN | LDR voltage divider (3.3 V → LDR → PA0 → 10 kΩ → GND) |
| PA10 | UART1 RX | Receives 'CAR' messages from ESP32 GPIO1 |
| 3.3 V | Power | Powers LDR voltage divider |
| GND | Ground | Common ground bus shared with camera and adapter |

## 9.5 Testing Plan & Recorded Results

| # | Test | Expected & Observed | Result |
| --- | --- | --- | --- |
| 1 | LDR exposed to bright light | DAY mode; all LEDs OFF | PASS |
| 2 | LDR covered | NIGHT mode; LEDs dim ~30% | PASS |
| 3 | Manual 'CAR' command in NIGHT | Wave propagates LED 1→9 then dims back | PASS |
| 4 | Manual 'CAR' in DAY mode | Command ignored; no wave | PASS |
| 5 | Object passes camera (NIGHT) | YOLO detects, wave fires within ~250 ms | PASS |
| 6 | Two objects within 1 s | Two overlapping independent waves | PASS |
| 7 | Stationary object in frame | One wave only; no repeats on same ID | PASS |
| 8 | Flash auto-control | ON in dim scene, OFF in bright scene | PASS |
| 9 | WiFi disconnect | ESP32 reconnects; controller continues | PASS |
| 10 | Cold boot | All subsystems online within ~4 s | PASS |

All ten test cases passed during the integration phase and were re-executed on the final hardware assembly prior to submission.

## 9.6 Demonstration Flow

- Show the maquette in normal room light: LDR senses DAY, all lamps off — system idle and saving power.

- Cover the LDR (or dim the room) to simulate dusk: lamps glow softly at dim ambient level.

- Open the laptop showing the YOLO detection window with the live camera feed.

- Pass a toy car (or any common object) along the road past the camera lens.

- YOLO bounds the object, fires a single 'CAR' signal, and a wave runs across the nine lamps.

- Pass a second object a second later: a second independent wave propagates without disturbing the first.

- Restore room light to show the system returns to DAY mode (lamps OFF) automatically.

# 10. Submission Checklist

## Completed Items

- ✅  STM32 Blue Pill firmware with multi-wave engine — complete.

- ✅  ESP32-CAM firmware with WiFi server, flash control, and /car endpoint — complete.

- ✅  Python YOLOv8 + ByteTrack detection pipeline — complete.

- ✅  Hardware schematic, breadboard wiring, and power design — complete.

- ✅  Maquette base with nine scale street lamps mounted — complete.

- ✅  Full 10-case test plan executed and all tests passed — complete.

- ✅  Project documentation (this document) — complete.

- ✅  Evaluation rubric annex filled in — complete.

- ✅  Final presentation deck — to be delivered with this submission.

## Future Work (out of scope for this submission)

- ○  Migration of host-side AI from laptop Python to a Raspberry Pi or NVIDIA Jetson (future work).

- ○  Training a custom YOLO model on real and toy vehicles to improve confidence on small targets (future work).

- ○  Outdoor weather-proof enclosure and mains-class power regulation (future work).

- ○  Replacement of WiFi with a wired or LoRa transport for deterministic outdoor operation (future work).

## Final Declaration

We confirm that the work presented in this documentation is our own original work, carried out collaboratively under the supervision of Dr. Amr Fahmy as part of the Embedded Systems course at the Arab Academy for Science, Technology & Maritime Transport. All external libraries used — Ultralytics YOLOv8, the Arduino core for STM32, and the ESP-IDF camera driver — are credited in the project repository.

— End of Documentation —